# Low Acrylonitrile NBR

## POLYMERIZATION / SYNTHESIS PATENT RESEARCH REPORT

### Abstract

The patent landscape for Low Acrylonitrile NBR (nitrile butadiene rubber) synthesis reveals a focus on emulsion polymerization processes. The major synthesis approach involves the use of various monomers, including 2-chloro-1,3-butadiene, sulfur, and optional monomers such as styrene, acrylonitrile, and methacrylonitrile. Recurring chemistry includes the use of rosin acids, metal salts of aromatic sulfonic acid-formalin condensates, and other emulsifiers. Trends in the patent landscape indicate a preference for temperatures between 30 to 55°C and conversions ranging from 70 to 90%. The resulting polymers exhibit improved properties, such as reduced heat build-up and enhanced abrasion resistance. However, some parameters, like acrylonitrile content and water amount, are not disclosed in the provided patent.

### Methodology

#### EP:3715409:B1 - Sulfur-modified chloroprene rubber composition, vulcanized product, molded article using said vulcanized product and method for producing sulfur-modified chloroprene rubber composition

Polymerization process: emulsion polymerization

Acrylonitrile content: Not disclosed

Monomer ratio: 2-chloro-1,3-butadiene and sulfur, optionally with other monomers such as styrene, acrylonitrile, methacrylonitrile, isoprene, butadiene, methacrylic acid and esters thereof

Water amount: Not disclosed

Emulsifier: rosin acids, metal salts of aromatic sulfonic acid-formalin condensates, sodium dodecylbenzenesulfonate, potassium dodecylbenzenesulfonate, sodium alkyl diphenyl ether sulfonate, potassium alkyl diphenyl ether sulfonate, sodium polyoxyethylene alkyl ether sulfonate, sodium polyoxypropylene alkyl ether sulfonate, potassium polyoxyethylene alkyl ether sulfonate, and potassium polyoxypropylene alkyl ether sulfonate

Initiator: potassium persulfate, benzoyl peroxide, ammonium persulfate, hydrogen peroxide

Chain transfer agent: Not disclosed

Temperature: 0 to 55°C, preferably 30 to 55°C

Reaction time: Not disclosed

Conversion: 60 to 95%, preferably 70 to 90%

Coagulation: Not disclosed

Polymer properties: Mooney viscosity of 20 to 80, improved permanent compression set and abrasion resistance, reduced heat build-up

### Cross-Patent Comparison & Synthesis Trends

#### Emulsifier Selection and Loading

The use of rosin acids, metal salts of aromatic sulfonic acid-formalin condensates, and other emulsifiers is a common trend in the patent landscape.

#### Initiator and Chain Transfer Agent Strategies

The patent utilizes potassium persulfate, benzoyl peroxide, ammonium persulfate, and hydrogen peroxide as initiators, while the chain transfer agent is not disclosed.

#### Monomer Ratio and Reaction Control

The monomer ratio involves 2-chloro-1,3-butadiene and sulfur, with optional monomers, and the reaction control is achieved through temperature and conversion management.

#### Coagulation and Post-Treatment Conditions

The coagulation conditions are not disclosed in the provided patent.

### References

|  Patent Number  |  Patent Title  |  Assignee  |  Jurisdiction  |  Publication Year  |  Google Patents URL  |

| --- | --- | --- | --- | --- | --- |

|  EP:3715409:B1  |  Sulfur-modified chloroprene rubber composition, vulcanized product, molded article using said vulcanized product and method for producing sulfur-modified chloroprene rubber composition  |  Denka Co Ltd  |  EP  |  2021  |  [https://patents.google.com/patent/EP3715409B1/en](https://patents.google.com/patent/EP3715409B1/en)  |