# Low-Styrene SBR Patent Landscape Report

## 1. Abstract

This report synthesizes three primary patents relevant to low-styrene styrene-butadiene rubber (SBR) synthesis and use. The target is defined as SBR embodiments with styrene content below conventional commercial grades, emphasizing reduced aromatic monomer fraction and butadiene-biased copolymer composition. Across the selected patents, two major synthesis families appear: aqueous emulsion polymerization for specialized E-SBR and solution/anionic polymerization for functionalized SBR. EP1967387B1 discloses an aqueous emulsion polymerization prepared E-SBR with 12–16% bound styrene, specifically positioned as a low-styrene elastomer for tire sidewall rubber compositions. US7612144B2 focuses on anionic solution polymerization of styrene/butadiene using sulfur-containing initiators, enabling functional head groups and improved filler interaction; the disclosed SBR examples contain about 21.7–22.2% styrene, which is not low-styrene by the target definition but is still relevant for process and end-group control. US8865829B2 discloses solution polymerization of styrene and butadiene followed by functional termination, with a comparative non-functionalized SBR and functionalized SBRs used in silica/carbon black compounds; the polymerization is conducted in hydrocarbon solvent at 65°C with conversion above 98%. Overall, the landscape shows that low-styrene SBR can be achieved through controlled aqueous emulsion polymerization with reduced bound styrene, while solution polymerization patents emphasize chain-end functionality, filler affinity, and rolling-resistance improvements rather than low styrene content itself. Formulation trends include use in tire sidewalls or tread compounds, reinforcement with carbon black and/or silica, and downstream mixing/cure protocols designed to translate polymer microstructure into lower hysteresis and improved dynamic performance.

## 2. Primary Patent Evidence

### Patent 1: EP1967387B1 — Rubber composition and tire sidewall with such a composition comprising …

| Patent Number | EP1967387B1 |
| --- | --- |
| Title | Rubber composition and tire sidewall with such a composition comprising … |
| Assignee | The Goodyear Tire & Rubber Company |
| Jurisdiction | EP |
| Publication Year | 2011 |
| Priority Date | Not available from source |
| Legal Status | Unknown |
| Polymer Type | Not available from source |
| Relevance Classification | PRIMARY |
| Relevance to Target | Automatically extracted candidate |

#### Polymerization / Synthesis Method

Synthesis method: The patent discloses a specialized emulsion-polymerization-prepared styrene/butadiene rubber (E-SBR) used in tire sidewall rubber compositions. The evidence states that the E-SBR is made by aqueous emulsion polymerization of 1,3-butadiene using an iron/hydroperoxide redox catalyst system, with controlled styrene/butadiene monomer charge to achieve the desired bound styrene level.

Bound styrene content: 14 percent — Example I

Tg (onset glass transition temperature): -65°C — Example I

Mixing temperature: 160°C — Example I

Mixing time: 4 minutes — Example I

Productive mixing temperature: 115°C — Example I

Productive mixing time: 2 minutes — Example I

Cooling temperature: below 40°C — Example I

NR phr: 10 to 40 phr — Claims

Specialized E-SBR phr: 5 to 35 phr — Claims

Cis 1,4-polybutadiene rubber phr: 50 to 70 phr — Claims

Particulate reinforcing filler phr: 30 to 80 phr — Claims

Bound styrene content: 12 to 16 percent — Claims

Tg range: -60°C to -70°C — Claims

Mooney viscosity: 60 to 100 — Claims

Preferred bound styrene content: 13 to 15 percent — Synthesis passages

Preferred Mooney viscosity: 70 to 90 — Synthesis passages

Medium & Water Role: The core polymerization medium is aqueous emulsion polymerization. Water is part of the polymerization medium for E-SBR preparation; no separate downstream water operation is emphasized in the extracted evidence.

styrene content below conventional SBR grades: 12 to 16 percent bound styrene; preferred 13 to 15 percent

#### Medium & Water Role

The core polymerization medium is aqueous emulsion polymerization. Water is part of the polymerization medium for E-SBR preparation; no separate downstream water operation is emphasized in the extracted evidence.

#### styrene content below conventional SBR grades

12 to 16 percent bound styrene; preferred 13 to 15 percent

#### Relevant Experimental Evidence

Example I: Demonstrated replacement of part of natural rubber with aqueous emulsion polymerization prepared E-SBR containing 14% bound styrene and Tg of -65°C in a sidewall compound.

Example I: Showed compounding of NR, cis 1,4-polybutadiene, E-SBR, carbon black, oil, wax, antioxidants, zinc oxide, sulfur, and accelerator.

Example I: Established that the specialized E-SBR is intended for tire sidewall formulations with low-temperature Tg matching NR.

Example I: Confirmed the E-SBR is an aqueous emulsion polymerization product made via an iron/hydroperoxide redox catalyst system.

#### Technical Relevance

This patent is directly relevant because it discloses a low-styrene E-SBR embodiment with bound styrene in the 12–16% range, which matches the target definition. It also links the polymer to tire sidewall performance, making it highly pertinent to low-styrene SBR synthesis and application.

### Patent 2: US7612144B2 — Use of sulfur containing initiators for anionic polymerization of monomers

| Patent Number | US7612144B2 |
| --- | --- |
| Title | Use of sulfur containing initiators for anionic polymerization of monomers |
| Assignee | Bridgestone Corporation |
| Jurisdiction | US |
| Publication Year | 2009 |
| Priority Date | Not available from source |
| Legal Status | Unknown |
| Polymer Type | Not available from source |
| Relevance Classification | PRIMARY |
| Relevance to Target | Automatically extracted candidate |

#### Polymerization / Synthesis Method

Synthesis method: The patent discloses anionic solution polymerization of styrene and butadiene using sulfur-containing initiators, including 2-lithio-2-methyl-1,3-dithiane and related lithio thioacetal initiators. The process is carried out in hydrocarbon solvent with polar modifier, and the living polymer is coagulated in isopropanol after polymerization; the main purpose is to introduce functional head groups and improve filler interaction rather than to minimize styrene content.

Reactor volume: 1.75 L — Example No. 2

Hexane: 1.12 kg — Example No. 2

Styrene solution: 0.48 kg of 33 wt % styrene in hexane — Example No. 2

Butadiene solution: 2.89 kg of 22.0 wt % butadiene in hexane — Example No. 2

Reaction temperature: 24°C — Example No. 2

Modifier: 0.5 mL of 1.6 M cyclic oligomeric oxolanyl alkane modifier in hexane — Example No. 2

Initiator: 22.63 mL of 0.234 M 2-lithio-2-methyl-1,3-dithiane in tetrahydrofuran — Example No. 2

Jacket temperature: 54°C — Example No. 2

Peak batch temperature: 76.5°C — Example No. 2

Reaction time after heating: 15 minutes — Example No. 2

Additional time before removal: 25 minutes — Example No. 2

Mn: 153 kg/mol — Example No. 2

Mw: 167 kg/mol — Example No. 2

Tg: -44.4°C — Example No. 2

Styrene content: 21.7% — Example No. 2

Block styrene: 1.3% — Example No. 2

Vinyl: 32.1% — Example No. 2

1,4 butadiene incorporation: 46.2% — Example No. 2

Reaction temperature: 50°C — Example No. 7

Reaction time: 4 hours — Example No. 7

Initiator concentration: 1.6 M butyl lithium in hexane — Example No. 7

Active lithium compound concentration: 0.234 M — Example No. 1

Synthesis temperature: -78°C — Example No. 1

Stirring time: 3 hours — Example No. 1

Overnight storage temperature: -25°C — Example No. 1

2-methyl-1,3-dithiane: 10 mL (83.5 mmol) — Example No. 1

Butyllithium: 55.83 mL of 1.510 M (84.3 mmol) — Example No. 1

1-bromo-3-chloropropane: 8.26 mL (83.5 mmol) — Example No. 1

THF: 350 mL — Example No. 1

THF for derivatization: 90 mL — Example No. 1

Polymerization initiator amount: 0.1 to 100 phm — Synthesis passages

Preferred initiator amount: 0.01 to 1 phm — Synthesis passages

Preferred initiator amount: 0.025 to 0.07 phm — Synthesis passages

Monomer ratio: 95:5 to 50:50 — Synthesis passages

Preferred monomer ratio: 95:5 to 65:35 — Synthesis passages

Medium & Water Role: No water-related process step disclosed in the extracted evidence.

styrene content below conventional SBR grades: Not disclosed in extracted evidence

#### Medium & Water Role

No water-related process step disclosed in the extracted evidence.

#### styrene content below conventional SBR grades

Not disclosed in extracted evidence

#### Relevant Experimental Evidence

Example No. 1: Prepared 2-lithio-2-methyl-1,3-dithiane and confirmed its structure by GC/MS.

Example No. 2: Produced poly(styrene-co-butadiene) with 21.7% styrene using 2-lithio-2-methyl-1,3-dithiane initiator.

Example No. 3: Showed a similar SBR with 22.2% styrene and slightly higher Mn/Mw using the same initiator.

Example No. 7: Demonstrated control polybutadiene synthesis and subsequent coagulation in isopropanol.

Example No. 12: Showed compounded stocks from functionalized polymers and reported reduced tan delta versus control.

#### Technical Relevance

This patent is relevant because it discloses solution/anionic SBR synthesis and the monomer feed, initiator, and temperature controls that govern SBR microstructure. However, the disclosed styrene levels in the SBR examples are around 21.7–22.2%, so the patent is only indirectly relevant to the low-styrene target.

### Patent 3: US8865829B2 — Functionalized polymer, rubber composition and pneumatic tire

| Patent Number | US8865829B2 |
| --- | --- |
| Title | Functionalized polymer, rubber composition and pneumatic tire |
| Assignee | The Goodyear Tire & Rubber Company |
| Jurisdiction | US |
| Publication Year | 2014 |
| Priority Date | Not available from source |
| Legal Status | Unknown |
| Polymer Type | Not available from source |
| Relevance Classification | PRIMARY |
| Relevance to Target | Automatically extracted candidate |

#### Polymerization / Synthesis Method

Synthesis method: The patent discloses solution polymerization of styrene and butadiene in a 1 gallon reactor at 65°C using n-butyl lithium and TMEDA, followed by termination with isopropanol or functional terminators. The resulting SBRs are then compounded with silica and/or carbon black to evaluate filler interaction and hysteresis, with the invention focused on functionalized end groups rather than low styrene content.

Reactor size: 1 gallon — Example 5

Reaction temperature: 65°C — Example 5

TMEDA: 5.7 mmol — Example 5

n-butyl lithium: 1.6 mmol — Example 5

Conversion: above 98% — Example 5

Mixing time: 4 minutes — Example 6

Mixing speed: 60 rpm — Example 6

Starting temperature: 140°C — Example 6

Compression molding time: 1 minute — Example 6

Second non-productive mix time: 3 minutes — Example 6

Productive mix starting temperature: 60°C — Example 6

Productive mix time: 3 minutes — Example 6

Silica: 65 phr — Table 2

Oil: 30 phr — Table 2

Stearic acid: 2 phr — Table 2

Zinc oxide: 3.5 phr — Table 2

Sulfur: 1.7 phr — Table 2

Sulfenamide: 1.6 phr — Table 2

Diphenylguanidine: 1.5 phr — Table 2

Antidegradant: 0.75 phr — Table 2

Polymerization temperature: 65°C — Example 5

Initiator amount: 1.6 mmol — Example 5

Medium & Water Role: No water-related process step disclosed in the extracted evidence.

styrene content below conventional SBR grades: Not disclosed in extracted evidence

#### Medium & Water Role

No water-related process step disclosed in the extracted evidence.

#### styrene content below conventional SBR grades

Not disclosed in extracted evidence

#### Relevant Experimental Evidence

Example 5: Demonstrated solution copolymerization of styrene and butadiene and termination at >98% conversion.

Example 5: Showed functionalized SBRs with different end groups and compared them to non-functionalized SBR.

Example 6: Established a silica-filled mixing protocol and measured Payne effect and tan delta.

Example 7: Showed that trithiocarbonate-functionalized SBR reduced strain dependency relative to control in non-productive mixing.

Example 10: Showed that trithiocarbonate-functionalized SBR lowered Payne effect and tan delta compared with non-functionalized control in productive mixing.

#### Technical Relevance

This patent is relevant to low-styrene SBR only as a solution-polymerization reference, because it discloses SBR synthesis and downstream compounding but not a reduced styrene target. Its main contribution is functional termination for improved filler interaction and rolling resistance.

### Patent Comparison Table

| Patent | Applicant | Medium & Water Role | styrene content below conventional SBR grades |
| --- | --- | --- | --- |
| EP1967387B1 | The Goodyear Tire & Rubber Company | The core polymerization medium is aqueous emulsion polymerization. Water is part of the polymerization medium for E-SBR preparation; no separate downstream water operation is emphasized in the extracted evidence. | 12 to 16 percent bound styrene; preferred 13 to 15 percent |
| US7612144B2 | Bridgestone Corporation | No water-related process step disclosed in the extracted evidence. | Not disclosed in extracted evidence |
| US8865829B2 | The Goodyear Tire & Rubber Company | No water-related process step disclosed in the extracted evidence. | Not disclosed in extracted evidence |

## Supporting / Related Patents (Secondary)

These patents are related to target synthesis but specific constraint relevance was not confirmed from available title and abstract data.

### Supporting Patent 1: US11130832B2 — Modified conjugated diene-based polymer and rubber composition including the …

| Patent Number | US11130832B2 |
| --- | --- |
| Title | Modified conjugated diene-based polymer and rubber composition including the … |
| Assignee | Lg Chem, Ltd. |
| Jurisdiction | US |
| Publication Year | 2021 |
| Relevance Classification | SECONDARY |

#### Technical Note

The patent concerns a modified conjugated diene-based polymer and its rubber composition, which is adjacent to SBR but not clearly the requested SBR base material itself.

### Supporting Patent 2: EP3359396B1 — Diene elastomer having a function in the middle of the chain and rubber …

| Patent Number | EP3359396B1 |
| --- | --- |
| Title | Diene elastomer having a function in the middle of the chain and rubber … |
| Assignee | Compagnie Générale des Etablissements Michelin |
| Jurisdiction | EP |
| Publication Year | 2019 |
| Relevance Classification | SECONDARY |

#### Technical Note

This is a diene elastomer functionalization patent based on anionic polymerization and post-modification, which is related to but not clearly about SBR as the central material.

### Supporting Patent 3: US9834619B2 — Modified conjugated diene-based polymer, modified rubber composition containing …

| Patent Number | US9834619B2 |
| --- | --- |
| Title | Modified conjugated diene-based polymer, modified rubber composition containing … |
| Assignee | Lg Chem, Ltd. |
| Jurisdiction | US |
| Publication Year | 2017 |
| Relevance Classification | SECONDARY |

#### Technical Note

The patent is about a modified conjugated diene-based polymer made by anionic polymerization and end-group coupling, but the packet does not establish that the central invention is SBR rather than a broader diene polymer platform.

### Supporting Patent 4: US20190112458A1 — Solution Polymerized Styrene-Butadiene Rubber-Polyurethane Elastomer Material …

| Patent Number | US20190112458A1 |
| --- | --- |
| Title | Solution Polymerized Styrene-Butadiene Rubber-Polyurethane Elastomer Material … |
| Assignee | Beijing University Of Chemical Technology |
| Jurisdiction | US |
| Publication Year | 2019 |
| Relevance Classification | SECONDARY |

#### Technical Note

The invention is primarily a polyurethane elastomer material that uses solution-polymerized SBR as a soft segment, so SBR is only a component rather than the central subject.

## 3. Cross-Patent Comparison & Synthesis Trends

EP1967387B1 is the only patent that directly discloses a low-styrene SBR target, with 12–16% bound styrene and a preferred 13–15% range; the other two patents disclose SBRs around 21.7–22.2% styrene or do not state a low-styrene embodiment.

EP1967387B1 uses aqueous emulsion polymerization with an iron/hydroperoxide redox catalyst system, whereas US7612144B2 and US8865829B2 use solution/anionic polymerization in hydrocarbon solvent.

US7612144B2 and US8865829B2 both emphasize chain-end functionality and filler interaction; US7612144B2 uses sulfur-containing anionic initiators, while US8865829B2 uses functional terminators after living anionic polymerization.

US7612144B2 discloses styrene contents of 21.7–22.2% in the SBR examples, which is substantially higher than the low-styrene range in EP1967387B1.

US8865829B2 discloses polymerization at 65°C with conversion above 98% and termination with isopropanol or functional terminators; its compounding studies focus on silica/carbon black reinforcement and hysteresis reduction rather than styrene reduction.

## 4. Conclusion

The strongest evidence for low-styrene SBR synthesis is EP1967387B1, which directly discloses an aqueous emulsion polymerization prepared E-SBR with 12–16% bound styrene, preferably 13–15%, and a Tg of -60 to -70°C. The other patents are valuable process references for solution-polymerized SBR and functionalization strategies, but their disclosed SBR examples are not low-styrene embodiments. For low-styrene SBR development, the evidence supports aqueous emulsion polymerization with controlled styrene feed and redox initiation as the most directly relevant route, while solution-polymerization patents suggest complementary approaches for end-group control and filler interaction optimization.

## 5. References

EP1967387B1 | Rubber composition and tire sidewall with such a composition comprising … | The Goodyear Tire & Rubber Company | EP | 2011 | https://patents.google.com/patent/EP1967387B1/en

US7612144B2 | Use of sulfur containing initiators for anionic polymerization of monomers | Bridgestone Corporation | US | 2009 | https://patents.google.com/patent/US7612144B2/en

US8865829B2 | Functionalized polymer, rubber composition and pneumatic tire | The Goodyear Tire & Rubber Company | US | 2014 | https://patents.google.com/patent/US8865829B2/en